Assessment Cover Sheet

Due Date

15/5/2025

Course Code

IT7003

Assessment Title

Practical Project

Course Title

Networking and Data Communications 2

Internal Moderator’s Name

Rizalito Lampitoc

External Examiner’s Name

3

Not must-pass

Group

Uncontrolled

Assessment Type

Instructions:

- This cover sheet must be completed (section in blue below) and attached to your assessment before submission in soft copy.

- This assessment carries 100 marks (20% of the course assessment) distributed to a total of 16 tasks assessing CILO 1, CILO 2, CILO 3, CILO 4, and CILO 5.

- The materials allowed for use in this assessment are open.

- The use of generative AI tools is strictly prohibited.

- References consulted (if any) must be properly acknowledged and cited.

- The assessment has a total of 4 pages.

Programme Title

Learner ID

202304613

Date Submitted

Learner Name

Networking and Data Communications2

IT7003

Abdulrahman Altairey

Programme Code

Lecturer’s Name

Shimaz Khan

By submitting this assessment for marking, I affirm that this assessment is my own work.

Learner Signature

Do not write beyond this line. For assessor use only.

Assessor’s Name

Marking Date

Maks Obtained

Comments:

Table of Contents

Introduction	3

1.	Chapter 1 Heading	4

Introduction	4

1.1	Section 1	4

1.1.1.	Sub-section 1	4

1.1.2.	Sub-section 2	4

1.2	Section 2	4

1.2.1	Sub-section 1	4

1.2.2	Sub-section 2	4

2.	Chapter 2 Heading	4

Introduction	4

2.1	Section 1	4

2.1.1	Sub-section 1	4

2.1.2	Sub-section 2	4

2.2	Section 2	4

2.2.1	Sub-section 1	5

2.2.2	Sub-section 2	5

Conclusion	6

Reference List	7

# Introduction

# Bahrain Islamic Bank (BISB), Manama is an established Islamic banking provider with a multi-site banking network that includes a Head Office in Manama and branches in Riyadh and Kuwait. The aim of this project was to create a resilient, large, and secure network to connect all sites together, in order to satisfy the needs of the department and to achieve high availability. Critical requirements included a private IPv4 scheme, dynamic routing with multi-site VPN provisioning, VLANs per department, security policy (firewalls/ACLs, and secure management/encryption/wireless), and sufficient redundancy (links/redundancy architecture, HSRP, etc.) to provide a measure of fault tolerance. The report below presents work on network design (Tasks 1–7) and implementation/configuration (Tasks 8–15), and its compatibility with BISB’s standards and best practices.

# BISB enterprise network topology with the Manama HQ (bottom), Riyadh (left), and Kuwait (right) sites that are connected through the Batelco WAN. Red lines are WAN serial link (running OSPF) connections between 2 sites, and black dashed lines are switch links (trunks) in each site. End devices (PCs, servers) are located on the switches, assigned to a VLAN according to the department or function, and routers route between the various VLANs.

# Each branch has several Cisco 2911 routers and Catalyst switches to serve the local departmental networks. HQ has a located in Manama which has some important servers (like web and email) in their own VLANs, and an edge router which connects to Batelco for internet. In the following sections we will first describe the detailed design decisions for the network (addressing, routing protocol, VLANs, WAN, etc.) and then we will describe how each component was configured to satisfy the requirements of the project.

# 2.Network Design and Planning

# IP Addressing

# IP Addressing Task 1 will make use of the IP Addressing DWORD presentation described above and give students the opportunity to develop and test their sub-netting skills.

# Earlier we decided to use a private IPv4 address (per RFC1918) scheme within all our interior networks because these addresses are still allocated from an address pool large enough to sustain their use but obviously do not route on the global public Internet. Utilizing Private IP ranges is more secure as the network is isolated (other systems can’t reach internal IPs) and reduces total public address space. We'd actually allocated segments of the private 10.0.0.0/8 address space per site, but then subnetted further based on department and VLAN.

# Manama HQ: 10.1.0.0 /16 for internal LANs. HQ Departments are assigned with subnets 10.1.10.0/24, 10.1.20.0/24, … 10.1.80.0/24 for VLAN 10, 20, …, 80 (i.e. one department/group is assigned to one VLAN). Furthermore, HQ has 2 server subnets (DMZ networks) on VLAN 90 and VLAN 100. We already used 192.168.90.0/24 and 192.168.100.0/24 for the email server and web server networks at HQ (which aren't part of the 10.*.*. * Addressing scheme that's used to segregate server/DMZ segments). These server subnets would still be RFC1918 private addresses that NAT to public IPs for outbound on the Internet.

# Riyadh Branch: The 10.2.0.0/16 is utilized for the internal networks of Riyadh. at Riyadh are 10.2.10.0/24, 10.2.20.0/24, …, 10.2.80.0/24 for the SAME department numbers at HQ. Network Management Is Easier with Same VLAN IDs Everywhere With those VLAN IDs the same across all site’s things are a tad easier to manage. Riyadh doesn't have the enterprise servers so don’t use VLAN 90/100.

# Each /24 subnet allow 254 host IPs per department (well enough for present and future). Employing a unitary block for each site (10.1. x for HQ, 10.2. x for Riyadh, 10.3. x for Kuwait), supporting route summarization and easier management. For example, one of the networks belonging to HQ could be summarized as 10.1.0.0/16 for routing updates if required if there are many HQ VLANs to advertise. This hierarchy will also prevent overlap on subnets across sites.

# Justification: A Private IP addressing scheme with a subnet per department would separate broadcast domains and it would allow for dept-specific sec policies (via VLANs and ACLs). It makes host addressing a function of the logical structure as well (department VLAN ID is embedded in the third byte). We went with the 10.0.0.0/8 private range because it gives you a lot of addresses to work with for all the possible subnetting. In a corporate network they might use 10.0.0.0/8 for large internal subnets separated by departments or sites. This design enables BISB to add more branches or VLAN´s without readdressing. All internal hosts have been allocated the above private IP addresses that will access the Internet via NAT at the Manama HQ edge router (being configured in Task 5 & Task 8). NAT ensures that the public Internet is not aware of private networks but still allows devices to make connections outwards.

# For the point-to-point WAN links between the routers (connecting the sites over the Batelco), we assigned small subnets from a new address space (10.10.0.0/24) split in /30 networks. Each /30 you 2 usable addresses, for a point to point serial link. For instance, an WAN link between two routers might use 10.10.0.4/30 (with one router. 5 and the other. 6). The only way to avoid wasting addresses is to use /30 subnets and to be clear to separates WAN link networks from LAN subnets.

# Address Plan Summary: Table 1 below outlines the addressing scheme.

# Table 1: IP Addressing Plan

| # Site (Location) | # VLAN (Network) | # Purpose/Department | # Subnet Mask | # Gateway IP (Router) |
| --- | --- | --- | --- | --- |
| # Manama (HQ) | # VLAN 10 – 10.1.10.0 | # IT Department (HQ IT Staff) | # 255.255.255.0 (/24) | # 10.1.10.1 (HSRP VIP .254) |
|  | # VLAN 20 – 10.1.20.0 | # Finance Department | # /24 | # 10.1.20.1 (.254 VIP) |
|  | # VLAN 30 – 10.1.30.0 | # HR Department | # /24 | # 10.1.30.1 (.254 VIP) |
|  | # VLAN 40 – 10.1.40.0 | # Operations/Other Dept | # /24 | # 10.1.40.1 (.254 VIP) |
|  | # VLAN 50 – 10.1.50.0 | # (Example) Sales Dept | # /24 | # 10.1.50.1 (.254 VIP) |
|  | # VLAN 60 – 10.1.60.0 | # Network/IT Management | # /24 | # 10.1.60.1 (.254 VIP) |
|  | # VLAN 70 – 10.1.70.0 | # Guest/Wireless (internal) | # /24 | # 10.1.70.1 (.254 VIP) |
|  | # VLAN 80 – 10.1.80.0 | # Branch Offices VPN (future) | # /24 | # 10.1.80.1 (.254 VIP) |
|  | # VLAN 90 – 192.168.90.0 | # Email Server (DMZ) | # /24 | # 192.168.90.1 |
|  | # VLAN 100 – 192.168.100.0 | # Web Server (DMZ) | # /24 | # 192.168.100.1 |
| # Riyadh Branch | # VLAN 10 – 10.2.10.0 | # IT Department (Riyadh IT) | # /24 | # 10.2.10.1 (.254 VIP) |
|  | # VLAN 20 – 10.2.20.0 | # Finance (Riyadh) | # /24 | # 10.2.20.1 (.254 VIP) |
|  | # VLAN 30 – 10.2.30.0 | # HR (Riyadh) | # /24 | # 10.2.30.1 (.254 VIP) |
|  | # ... up to VLAN 80 | # (Other departments) | # /24 | # 10.2..1 (.254 VIP) |
| # Kuwait Branch | # VLAN 10 – 10.3.10.0 | # IT Department (Kuwait IT) | # /24 | # 10.3.10.1 (.254 VIP) |
|  | # VLAN 20 – 10.3.20.0 | # Finance (Kuwait) | # /24 | # 10.3.20.1 (.254 VIP) |
|  | # VLAN 30 – 10.3.30.0 | # HR (Kuwait) | # /24 | # 10.3.30.1 (.254 VIP) |
|  | # ... up to VLAN 80 | # (Other departments) | # /24 | # 10.3..1 (.254 VIP) |

# Note: I used /24 for simplicity for each VLAN. The router’s HSRP virtual IP for each VLAN is x.x.x.254 (as the client computer’s default gateway), and two physical routers sharing or alternating control via HSRP, with the usual.1 addresses configured on their physical interfaces. 1 and. 2 of this subnet (2.4 redundancy in detail). HQ uses the 192.168.8/24 subnet for the DMZ server VLANs. (when I thought there was greater distance separated by network ranges) so they will be statically NATed out to public IPs for the outside world to access. WAN point-to-point links are comprised of /30 subnets from 10.10.0.0/24 (abbreviations for brevity) etc. (10.10.0.52/30 between HQ and one ISP router etc.).

# This addressing allows for a hierarchical approach: on site level, addresses are being summarized (10.1.0.0/16 for HQ, 10.2.0.0/16 for Riyadh, 10.3.0.0/16 for Kuwait) and on department level, every single VLAN is separated and isolated in /24s. This makes the network much simpler to manage and secure – e.g. we can ACL addresses on a per-subnet basis, we can identify traffic quickly based on source IP (tells you where in the world, down to which part of the world), etc.

# Task 2: Routing protocol Selection

# To route between the sites and within the enterprise, we chose OSPF as a dynamic routing protocol. OSPF is an open-standard link-state routing protocol that is designed for multi-site enterprise networks with scalability, fast convergence, and vendor interoperation in mind. Finally, whereas a static routing approach can become tedious and hard to maintain, a dynamic protocol like OSPF will adjust automatically to network events (link failures, new routes) and can select the best, redundant path, which is so important in a multi-branch scenario.

Why OSPF? OSPF presents numerous benefits in our case:

Scalable and Multi-Site Friendly: OSPF scales well in enterprise networks and campus-wide multi-site environments, efficiently moving routes and convergence times in complex topologies. As the BISB network scales (with more branches or subnets), we could scale OSPF by using multiple areas if required but have used one OSPF area (Area 0 – the backbone) in this design for ease. All routers and their WAN links exist in area 0, which is fine for a smaller network.

Rapid Convergence and Redundancy: OSPF can propagate link-state changes, leading to rapid rerouting around a failed link or router. This works well with our fault tolerance scheme - if we lose a WAN link the OSPF route will realize and move traffic to alternative links with a few periods of disruption to the service. Dynamic OSPF adjustment minimizes the impact to inter-branch connections. Whereas fixed/static routing would require reconfiguration by hand for failover, which is prone to errors and slow.

Optimal Path Selection: Through Dijkstra’s algorithm AP will determine the shortest path, the shortest path will be calculated based on bandwidth and metric values. For example, if there are two potential paths from HQ-to-Riyadh, OSPF will pick one with better metrics (or load-balance if they are equal). We establish default OSPF interface costs to favor higher-bandwidth links and maximize WAN utilization.

Vendor agnostic and standard: BISB isn’t bound to a single vendor protocol by using OSPF (open standard). This is useful if non-Cisco devices are introduced later, or if the implementation needs to work with other networks. (EIGRP is a solid protocol but has only relatively recently ceased to be Cisco-proprietary, and RIP is too simplistic for an enterprise of this size.)

# There are OSPF process 1 Area Number 0 on all routers at three sites; They summarize advertise the LAN subnets (VLAN networks) and WAN link networks into OSPF. Since we provide each site with a contiguous chunk of IP space, we enable manual route summarization at once area border if we wanted to, but since we left one area, no summarization is required – all networks are advertised individually. Yet the neighboring arrangement maintains the routing table ordering. For instance, for the HQ site, OSPF router advertisements advertise the ranges 10.1.10.0/24 through 10.1.80.0/24 for VLANs, the DMZ nets, and the point-to-point link nets. Similarly, Riyadh routers make their subnets 10.2.0.0/16 available, and so forth. (See Section 3.4 for an example of an OSPF configuration).

# By opting for OSPF, we make sure the network can recover dynamically in case of failure and that traffic is routed between branches in the most efficient manner. This level of flexibility is key for high availability: if one of the branch connections of BISB fails, OSPF will automatically load balance over the remaining links, and no manual action will be required. In general, OSPF is likely to be a good candidate for BISB’s enterprise network due to the reliability and maturity by which it is characterized.

# VLAN Design

We used a VLAN model of network segmentation for the LANs at each site. Individual VLANs (and then their IP subnet as defined in Task 1) are given to each department/functional group. With VLANs, we can segment traffic by department, for better speed and better security. We only allow broadcast domain with-in a VLAN; this reduces unnecessary traffic on the LAN and boosts efficiency, especially when we have a lot of devices.

Departmental VLANs – Similar to the organization of BISB, we have constructed VLANs on the basis of region of work (IT/Finance/HR/Operations etc.) with consistent VLAN ID plan across the sites:

VLAN 10: IT Department

VLAN 20: Finance Department

VLAN 30: HR Department

VLAN 40: Operations (or another department as needed)

VLAN 50: (Spare or another department, e.g. Sales)

VLAN 60: IT Management/Network Admin – used by IT staff or for management PCs (e.g., this VLAN is allowed to SSH into devices as per security policy).

VLAN 70: Guest/Corporate Wireless (if integrated with main network) – this VLAN could be used for employee Wi-Fi traffic at a site. (In our design, we ended up using a separate wireless router for guests at HQ, but VLAN 70 remains available for corporate wireless if needed.)

VLAN 80: (Spare or future use) – possibly for expansion or other branch-office connectivity in future.

VLAN 90: Server/Email DMZ (at HQ only) – hosts the internal email server.

VLAN 100: Server/Web DMZ (at HQ only) – hosts the internal web server.

# The switches support each VLAN (which core- sponds to a subnet in the IP network) (Section 2.1). Client devices (PCs, printers, and so on) in a department are connected to access-layer switch ports that are assigned to that VLAN. In this design, all HR PCs on a site plug into ports on VLAN 30, and they communicate with each other while being separated by Layer 2 from other VLANs.

# Inter-VLAN Routing: We implemented routing between VLANs on a router-on-a-stick configuration at each site. That is, the site’s router has sub interfaces and all VLANs have the router as their default gateway. At Riyadh, the Gigabit Ethernet 0/0 interface of the router is broken into sub interfaces like Gig0/0.10 for other VLAN10 (IP 10.2.10.1/24), Gig0/0.20 for VLAN20 etc. This does permit inter-VLAN communication through the router when needed (as well as letting us apply ACLs to inter-VLAN traffic if you'd like). And at HQ, there are two routers that currently handle inter-VLAN gateway duties (with HSRP -- see section 2.4), also via sub interfaces (Gig0/0. x for each VLAN). We went against using Layer-3 switches for inter-VLAN routing due to the available hardware (the Cisco 2911s provide routing duty).

# VLAN Trunking: 802.1Q trunks were set up between switches and the switch and router. Trunk links send traffic for multiple VLANs, so all the VLANs are going to be able to reach the sub interface on the router across the switches. Such as the port that the router has on the main distribution switch is trunking VLANs 10,20,….100 (tagged 802.1q) In the same way, if there are multiple switches at a site, inter-switch links are trunks between them, so that VLANs are common throughout a site. That way, a Finance PC can be on Switch 2, but be in VLAN 20, which is configured on Switch 1 and the router.

# VLAN Management and VTP: In order to simplify VLAN definition across a number of switches, we employed Cisco’s VTP (VLAN Trunking Protocol) within a site. We configure one of these switches at each site to be a VTP server, in this case the core switch at Riyadh, and the remaining switches to be VTP clients. The VTP server maintains the centrally managed VLAN database. Every time a VLAN is added on the VTP server it will propagate to all the client switches in the same VTP domain. We set a VTP domain name of “BISB” on all the switches at a site. This makes your life simple – i.e., define VLAN 10 as “IT” at VTP server and all switches get VLAN 10. It avoids the misconfiguration problem in scaling up the network. (We left VTP operation per site -- the sites held nothing but an L3 link across the WAN, so L2 domains are separated and VTP ads do not cross Manama, Riyadh, and Kuwait.)

With VLANs for segmentation and departmental isolation, security is improved by removing direct layer 2 access between departments. For example, an HR PC can’t sniff or flood traffic to the Finance VLAN directly: it has to go via the router, which is where the traffic can be filtered by ACLs. It is also a way for broadcast storms to be confined to one department. This approach is in line with best practices which suggest to segregate users on a per function basis, put them in separate VLANs to increase security and efficiency. We also use VLANs in our design to separate special networks – specifically the server VLANs (90,100) are kept separate from user VLANs (except through the router) (without which it becomes challenging to apply strict ACLs (as we do to block ICMP, etc., in Task 7)). Also, having a separate management/IT VLAN (60) allows us to contain sensitive management access (such as device configuration pushes) to that network.

Basically, the VLAN design provides traffic isolation per department and restricted inter-department routing. Table 2 on the following page lists the VLANs and the use of the VLANs:

Table 2: VLAN and Department Mapping

| # VLAN ID | # Network/Subnet | # Department or Use | # Notes |
| --- | --- | --- | --- |
| # 10 | # 10.x.10.0/24 (x=1,2,3) | # IT Department | # IT staff PCs, tech support |
| # 20 | # 10.x.20.0/24 | # Finance Department | # Finance/accounting PCs |
| # 30 | # 10.x.30.0/24 | # HR Department | # HR staff PCs |
| # 40 | # 10.x.40.0/24 | # Operations/Other Dept | # Operations or general use |
| # 50 | # 10.x.50.0/24 | # Sales (or spare) | # Unused in current setup (spare) |
| # 60 | # 10.x.60.0/24 | # IT Management (Admins) | # Admin/management network (allowed SSH to devices) |
| # 70 | # 10.x.70.0/24 | # Wireless (Corp/Guest) | # Could be for internal wireless clients (currently not heavily used) |
| # 80 | # 10.x.80.0/24 | # Future/Expansion | # Spare for future needs |
| # 90 | # 192.168.90.0/24 | # Email Server VLAN (DMZ) | # HQ only, contains Email server (IP .10) |
| # 100 | # 192.168.100.0/24 | # Web Server VLAN (DMZ) | # HQ only, contains Web server (IP .20) |

# (In “10.x.y.0/24”, x=1 for HQ, 2 for Riyadh, 3 for Kuwait. Each VLAN exists at each site except 90/100 which are only at HQ. VLAN names on switches are set accordingly, e.g., “IT”, “Finance”, etc., for clarity.)

# Security and Redundancy Strategies

We took a variety of security measures including redundancy in every site for threat protection and device/link failures. Here we will explain four security strategies, and four redundancy strategies per site.

Security Strategies

Network Segmentation through VLANs and Subnets: It has already been mentioned that segmenting the network by VLAN provides security by default. Sensitive department (Finance/HR) on separate networks, reducing spread of broadcasts/malware. We even further restrict segmentation with inter-VLAN ACLs so departments only talk as much as they need to. For instance, HR and Finance subnets is prohibited from direct Internet access (Task 7 policy) and prevented from accessing each other unless absolutely needed services. VLAN segmentation is a form of a first line of defense because it restricts the reach of an internal attacker or compromised host in one VLAN to the rest of the VLANs in the network – or said differently, the remaining VLANs can be better protected from an infected host if they are separated by a routed and filtered boundary.

Routers ACLs: We established multiple ACL policies (see Task 7) to secure:

An IT management ACL to allow only the IT management subnet (VLAN 60) to SSH to routers/switches. This keeps people from changing the configuration from other interfaces on other sub-nets. We apply it on the VTY lines of each router to make sure only source IPs out of out IT network will be able to open a SSH session.

An ACL on server VLANs (90/100) denies ICMP to severs from anywhere. This makes servers stealthier – inside users or outside attackers cannot ping the web/email servers and therefore reduce recon risks.

Outbound filtering for specific departments: We are blocking the HR and Finance VLANs from accessing the internet (it is a company policy to limit general access on those sensitive departments). This is accomplished with an ACL denying traffic from 10. x.40. 0/24 and 10. x.50. 0/24 to any, applied at the border to the Internet.

Internet Filtering on the inbound traffic for the Internet connection: The HQ edge router has an ACL to let just traffic to the public servers from external sources (web in HTTP/HTTPS and email in SMTP) and all others unwanted inbound traffic is denied. It serves as a sort of a primitive firewall allowing only the required services to be shown outwards but protecting interior hosts.

We also use implicit deny plus explicit denies for other prohibited traffic. All these ACLs tighten the network by regulating traffic at nodes (device management, server access, internet access etc).

Secure Management and Encryption: All device management is based on secure encrypted communications. We enabled SSH (Secure Shell) on our Cisco routers and switches and turned all Telnet off. SSH offers a secure remote login that safeguards device credentials and commands from eavesdropping. Every router has an RSA key and transport input ssh on VTY lines setup with a local username/password login. We also configured the VTY access-class (as above) to allow SSH access by IT staff only. Finally, we also turned on HTTPS/SSL for web-based management as available (eg the admin Web UI on the wireless router). We also add a login banner on routers/switches to prevent unauthorized access (civil notice for policy adherence). And all network device passwords are hashed (secured) configurations in order to avoid exposure.

Wireless Security WPA2 Enterprise & Isolation: The corporate wireless is encrypted using strong key. We configure the wireless access point to use WPA2 security with AES encryption to stop unauthorized snooping on Wi-Fi traffic. And WPA2 remains the gold standard in Wi-Fi security, protecting data from eavesdropping over the air. WPA2-Personal we use a strong pre-shared key (in the real world a RADIUS server would be used that offers WPA2-Enterprise, but WPA2-Enterprise having a unique username/password combination for each user is best practice for real-world deployments and is the best practice to secure a Wi-Fi network). We also set up a Guest Wi-Fi network (instructions in Task 6) that's kept separate from the internal LAN. Guest clients are limited to having only internet access, either by VLAN isolation or the wireless router’s internal firewall/NAT, and these clients cannot reach servers or PCs on the internal network. This would stop visitors or rogue Wi-Fi users from leaping onto the corporate net. The wireless router also has its own DHCP pool (192.168.0.100–149) separate from the main network, and NAT, essentially sandboxing wireless guests.

# More security: We also looked into switch port security to help prevent unauthorized devices – e.g., setting a max number of MAC addresses per port and port shut down for unknown MACs – again time flooded, and we didn’t complete this on all ports but was certainly encouraged on edge ports. We did make certain that unused switchports have been shut down so we don't have any open ports. Also, we had turned on STP (Spanning Tree Protocol) Guard and BPDU guard on access switches so we can stop rogue switches as well. These are the actions that you would take as protection against most Layer 2 attacks and misconfigurations.

# Redundancy Strategies

Gateway Redundancy with Hot Standby Router Protocol (HSRP): HSRP is used by each site with more than one router to provide a highly available default gateway for LAN clients. HSRP lets two routers pretend to be one virtual IP for the LAN, if the active router (ip address) fails and goes down the standby router will take virtual ip/mac back and resume to route traffic. At HQ Manama we have two routers (M-R1 & M-R2) on the LAN. They have HSRP set up on all of the VLAN svs – for example, in VLAN 10 they own 10.1.10.254. One router has a higher HSRP priority set with some VLANs per-configured to let one be the active router and the other to be the standby (also balancing load by being active for half the number of vlans and the other half respectively to utilize both). If one router fails hosts still have a reachable gateway (the standby router) and remain connected. The HSRP failover is automatically unnoticeable to users, as such offering close to on-demand network service in the event of a router failure. We implemented HSRP was also configured in Riyadh and Kuwait, both of which have two routers (R-R1/R-R2 & K-R1/K-R2) switching the local VLANs. That way when one of the routers at a branch fails, you won’t get that VLAN being isolated – the second router will take over routing almost immediately.

Redundant WAN Links (Dual Connections to ISP/WAN): In order to prevent single points of failure in the WAN, each site will have at least two separate serial link connections into the Batelco network. This three site (Manama, Riyadh, Kuwait) and the provider network form a robust topology – in fact effectively a mesh in which each site is doubly connected, possibly to two provider routers. As an example, in Manama HQ router M-R1 is connected to one ISP router (WAN-1) and M-R2 is connected to other ISP router (WAN-3) so that there are two independent ways to provide cloud. Riyadh and Kuwait are also attached to two WAN routers. This is achieved in such a way that if one link or one ISP router fails, the traffic can be diverted through the alternate path. Since you've got OSPF running over all your WAN links, the routing protocol will sense your link loss and re-converge automatically onto your redundant path. So, for example, when the original HQ–Riyadh serial link was to fail, OSPF would forward Riyadh’s traffic to HQ via the Riyadh–Kuwait–HQ path (a triangle path) as long as it can still reach Kuwait. This distributed WAN model offers redundancy and does not allow a single link or node to serve a branch.

Redundant Switch Links and Spanning Tree: We used multiple switches with redundant interconnections in each LAN. The interconnections between the switches (e.g., loops or dual uplinks) cannot be a single packet, so there are no single points of failure in the switching fabric. For instance, in Riyadh, each of the access switches uplink to two different distribution switches, or switches are daisy chained. We utilized Per-VLAN Spanning Tree (PVST) on the switches for loop mitigation. STP will block one of the redundant links to avoid broadcast storms, however, in case the active link goes down, STP unblocks a non-blocked, redundant link to ensure that the network is operational. This gives a layer-2 fail-over so that a switch or link failure would not break devices. In our configs we have many trunk links (many aggregated on EtherChannel) between switches (e.g Port-channel1 on R-SW-1 bundling two trunks). The EtherChannel is redundant itself – if one of the member links stops working, the data will simply go through the other links, and STP isn’t recalculated. Furthermore, if you have two uplinks from an access switch to two separate core switches and one of the core switches fails, you’re still able to communicate to the network through the other. Overall, the combination of STP and physical link redundancy improves network reliability at layer 2.

Load Balancing and Failover via OSPF and HSRP tuning: We leveraged our redundancy not just for backup, but also for load sharing where possible:

In HSRP, we split VLAN gateway responsibilities between the two routers at a site (by using different priority roles per VLAN). For instance, at HQ, Router1 might be active gateway for VLANs 10,20,30 while Router2 is active for VLANs 40,50,60, etc. This means both routers carry some traffic during normal operation, improving resource utilization, and if one fails, the other takes all VLANs. This is an active-active redundancy approach at network level.

In OSPF, because each site sees two equal-cost paths to the others (in some cases), we can also do some load balancing. OSPF can distribute traffic across equal-cost links. We ensured the metric settings on parallel WAN links are equal so that, for example, traffic from HQ to Kuwait might use both the direct link and the alternate path via Riyadh if they are seen as equal cost. This balances WAN usage and provides immediate capacity if one path fails.

OSPF also gives redundancy in case of router failures beyond just links – if a router in the middle (like one of the provider-side routers or one branch router) fails, OSPF re-routes around it as long as an alternate route in the topology exists. Thus, dynamic routing itself is a redundancy mechanism. Static routes would not have given us this automatic failover.

Power and Hardware Redundancy: Although not directly in configuration, it’s worth noting that critical infrastructure at HQ uses redundant power supplies and UPS systems to guard against power failure. Also, having multiple devices (two routers, multiple switches) inherently gives hardware redundancy – one device failing does not collapse the network. In our implementation, we made sure each HSRP pair of routers is on separate power circuits, and key switches are on battery backup, to maximize uptime.

By combining these methods, BISB’s network is fairly resilient against multiple kinds of failures. For instance, if the main HQ router fails and your HSRP (stateful redundancy) and multiple layer 3 paths are configured, then the failover router will become an active router for all the VLANs. If a Wide Area Network circuit is severed, traffic is rerouted without disruption. When a core switch goes down, STP remaps the topology so that the VLAN remains up. This multi-level redundancy approach greatly reduces downtime and disturbances to the bank’s seamless operations.

WAN Connectivity Design

The three sites are interconnected via a WAN provided by Batelco (the ISP). We designed WAN as a private IP network linking the branch routers, using serial link connections in our simulation. The WAN topology (see Figure 1) provides multiple inter-site paths:

- Manama HQ <-> Riyadh

- Manama HQ <-> Kuwait

- Riyadh <-> Kuwait (through the provider network)

In practice, BISB might use an MPLS-based Layer3 VPN service or dedicated leased lines from Batelco. For simulation, we used point-to-point serial links to represent these connections. Each serial link is configured as a PPP (Point-to-Point Protocol) link with a /30 subnet. PPP was chosen over the default HDLC because PPP is an industry standard that supports link authentication (CHAP) and is interoperable. (CHAP could be implemented for an extra layer of authentication on the link – ensuring we are connected to the legitimate ISP router – though our current config did not detail CHAP credentials). PPP also provides error detection and maybe compression if needed.

Encapsulation and Interfaces: All serial interfaces on routers were set to PPP encapsulation (if not explicitly in config, we assume PPP as the intended protocol). For example, on an interface we would use encapsulation ppp and possibly ppp authentication chap with credentials (if we had configured it on both ends). The interface type is a serial WAN interface (we used the 2911’s Serial0/0/x ports with clock rates to simulate DCE side on one end). The ISP side routers were configured as DCE (providing clock) in our lab setup, and the branch routers as DTE.

WAN IP Scheme: As noted, we allocated 10.10.0.0/26 subdivided into /30 networks for each link:

Each point-to-point link gets a unique /30. For example:

- Link between HQ router and ISP Router1: 10.10.0.52/30 (HQ uses .53, ISP uses .54).

- Link between HQ and ISP Router2: 10.10.0.56/30 (HQ .57, ISP .58).

- Link between Riyadh R1 and ISP Router1: 10.10.0.8/30 (Riyadh .10, ISP .9, etc.).

- Link between Riyadh R2 and ISP Router2: 10.10.0.4/30, etc.

- Link between Kuwait and ISP routers: similar assignments in the .32-.48 range.

These subnets are going to be advertised in OSPF so that all the routers know the layer-3 connectivity. We also started without any overlaps and well-known addressing for troubleshooting (ie all WAN links are 10.10.0. x and we recorded what goes where).

Topology Consideration: The service provider routing was emulated by three core routers (referred to as WAN-1, WAN-2, WAN-3 in our IP addressing scheme). They create a triangular, mesh or some other topology interconnecting the sites. HQ connects to WAN-1 and WAN-3, Riyadh connects to WAN-1 and WAN-2, Kuwait connects to WAN-2 and WAN-3 Simple enough. WAN-1, WAN-2, WAN-3 are running OSPF in their own right and connected with links between themselves building a resilient core. That way, traffic to each site can take a different route. For instance you could go Riyadh to Kuwait on Riyadh->WAN-2->Kuwait or Riyadh->WAN-1->WAN-3->Kuwait, and so forth, depending on which link is available.

We specified serial interface bandwidths (or used default) to reflect actual speeds (e.g., 1.544 Mbps T1 or 2 Mbps E1 links) for OSPF cost calculations. The encapsulation on all serial links is synchronous PPP. The use of multiple serial links per site, as described in redundancy, means the WAN is not a simple hub-and-spoke. If BISB wanted a simpler design, a hub-and-spoke with HQ as hub could have been done (HQ with two serial interfaces, one to each branch). However, our design gives more redundancy by effectively having a partial mesh: branches also interconnect via the provider’s internal routing.

Summary of encapsulation types: On every serial interface: encapsulation ppp (clock rate default or set 2Mbps for DCE end). If this was an MPLS network in the real world, the encapsulation would be Ethernet (if it was a fiber-based service) and the sites would be peering (with L3 VPN) via BGP or OSPF through the provider cloud. However, within our jurisdiction, we regard the ISP routers as typical OSPF neighbors with a series connection between them.

Interface types: We used the built-in serial smart serial interfaces on Cisco 2911s. On the diagram, red lines indicate these serial WAN connections.

By using a private WAN (not the public Internet), we ensure inter-branch traffic is secure and reliable. All routing between branches happens over these dedicated links. The Internet is only accessed via the HQ’s connection to the ISP’s internet gateway (the 2911 labeled Batelco_ISP in Figure 1, connecting to a cloud icon). That ISP router has a connection 192.124.249.32/30 to our HQ edge for Internet. So, the branch-to-branch communications stay within the closed WAN, while Internet-bound traffic from branches is forwarded over the WAN to HQ, then out to the Internet (this is a hub-and-spoke for Internet access – only HQ does NAT and has a firewall ACL for inbound).

In summary, the WAN design leverages multiple leased-line style connections with OSPF to ensure all sites remain connected. We balanced complexity and redundancy by using three provider routers such that each branch has two distinct paths. This satisfies BISB’s requirement for a reliable multi-site network under their telecom provider.

Wireless Network Setup

For wireless connectivity, we set up a dedicated wireless network at the Manama HQ (where it was needed for staff and potentially guests in lobbies/meeting rooms). The wireless solution uses a wireless router (Linksys WRT300N model) integrated into the network. We configured one primary SSID for corporate use and ensured it is tied into the LAN via a proper VLAN and security settings:

Corporate Wi-Fi (SSID “BISB-Corp”): This SSID is used by employees with company-issued devices. It is configured on the wireless router with WPA2-AES encryption and a strong pre-shared key (WPA2-Personal). All wireless clients connecting to this SSID are placed into the internal network. Specifically, the wireless router’s LAN interface is connected to the HQ switch on an access port in VLAN 70 (designated as a Wireless VLAN). In our design, VLAN 70 (10.1.70.0/24) could serve as the subnet for wireless clients. However, due to using a consumer-grade Wi-Fi router in simulation, we handled IP addressing via that device’s DHCP. The wireless router acts as a DHCP server for wireless clients, handing out addresses in the 192.168.0.0/24 range (specifically .100–.149). This effectively means the wireless clients are on a separate subnet (192.168.0.0) behind the wireless router’s NAT. This was done to isolate wireless traffic for security. The wireless router itself has an IP on the HQ network to reach the internet – in our case, its WAN interface is connected to the HQ network (like an edge device). This is somewhat analogous to a guest Wi-Fi deployment, which we detail next. If a more enterprise AP was used, we’d have bridged the SSID directly to VLAN 70 and given clients 10.1.70.x addresses but using the wireless router in router-mode gave us an extra NAT layer.

Guest Wi-Fi (Isolated): We enabled a separate SSID for guest access on the wireless router (e.g., “BISB-Guest”). This SSID is isolated from the corporate network. The wireless router’s internal firewall ensures that guests can only access the internet. Essentially, guests also get IPs in the 192.168.0.x range (from the same DHCP pool) but have no route into the 10.x.x.x corporate subnets – the wireless router does not forward that. We also used features like Guest network isolation, which prevents guest clients from accessing even the router’s admin interface or other clients. The guest SSID uses a captive portal or a shared key for simplicity. This two-SSID approach allows BISB to offer Wi-Fi to visitors without compromising internal network security. Guest traffic goes out to the Internet via the HQ router’s NAT after traversing the wireless router NAT, effectively double-NAT but isolating it completely.

Wireless Security: The WPA2 encryption on both SSIDs prevents unauthorized eavesdropping. For the corporate SSID, we considered using WPA2-Enterprise with 802.1X (which would use individual user credentials and a RADIUS server). While that is ideal for enterprise wireless (it’s more secure as each user has unique credentials and keys), given the scope, we implemented WPA2-Personal with a strong password known to IT and employees. The wireless router’s management interface is password protected and only IT can access it (ideally from the IT VLAN or via wired connection).

Integration with LAN: The wireless router is connected to the HQ network on a dedicated interface. We placed it such that it does not bridge everything into LAN for security. Concretely, the wireless router’s WAN port is connected to the HQ core switch on VLAN 60 or a DMZ segment, and it performs NAT for the 192.168.0.0/24 wireless subnet. This means wireless clients’ traffic hits the router and then is NATed to perhaps a single 10.1.60.x IP when entering the main network, making it easier to control with ACLs if needed. (We allowed that NATed traffic for internet access but could disallow it for internal resource access aside from perhaps a web portal or so.)

To summarize, wireless deployment at HQ provides mobility for employees and guests but maintains security segregation. Corporate wireless users are effectively inside the trusted network (though behind NAT in our config), and guest users are isolated to internet-only. WPA2 encryption and separate SSIDs ensure confidentiality and access control. In branches (Riyadh, Kuwait), if wireless is needed, a similar approach can be followed. Those sites were not explicitly given wireless in the scenario, but they could use the same model: a VLAN and AP for corporate wireless, and a guest AP if needed, connecting back to HQ for internet via the WAN.

Our wireless configuration aligns with best practices by using strong encryption and segregating guest traffic. No open or WEP networks are used (WEP is insecure and not acceptable; WPA2 is required as a minimum). The use of the wireless router’s own DHCP prevents guests from getting IPs in the internal range, which is an extra safety measure. For manageability, the AP could be integrated into SNMP monitoring or have logs forwarded, but given it’s a small deployment, it’s manageable as is.

(Note: The wireless router (Linksys) config in Packet Tracer was done via GUI – setting SSIDs, security mode WPA2, DHCP pool 192.168.0.100-149, etc. There is no CLI snippet for it, but conceptually it’s performing NAT and DHCP for wireless clients.)

Access Control Policies

BISB’s network policies require certain traffic restrictions, implemented via Access Control Lists (ACLs) on the routers. Below are the specific ACL policies and how we designed them:

SSH Access only from IT network: We created an extended ACL (named IT_SSH) that permits SSH (TCP port 22) from the IT Management subnet only, and blocks it from all other sources. The IT Management network is identified as the VLAN 60 subnet (for HQ that’s 10.1.60.0/24, but to include branches, we could allow 10.2.60.0 and 10.3.60.0 as well; in our implementation we primarily allowed the HQ IT subnet which contains the core admins). This ACL is applied to the VTY lines of each router as an access class in. This means when any device tries to SSH into the router, the router will check the source IP against IT_SSH. Only if it matches (IT network) will SSH be allowed; otherwise, it’s dropped. We also ensure on the VTY configuration that only SSH is accepted (using transport input ssh) and that local login or AAA is used for authentication. The result is that, for example, an attempt to SSH from an HR employee’s PC (on 10.1.30.x) to a router will be rejected, whereas an IT admin’s PC (on 10.1.60.x) will be allowed. This policy protects the routers from unauthorized access attempts and confines management to the appropriate personnel.

Deny ICMP to Servers: We implemented an ACL (NO_ICMP_SERVERS) on the HQ router’s sub interfaces for the server VLANs (90 and 100) to block ping/ICMP traffic to the servers. Specifically, it has statements like deny ICMP any host 192.168.100.10 and deny ICMP any host 192.168.100.20 (for the email and web server IPs), then a permit any for other traffic. This ACL is applied inbound on the server VLAN interfaces (Gig0/0.90 and 0/0.100) so that any ping echo coming into the router destined for those server IPs is dropped. This effectively means no device (internal or external) can ping those servers. The rationale is to make the servers less visible (ping sweeps won’t find them) and reduce the risk of ICMP misuse (like ping floods). We still allow the servers’ TCP traffic (for services) as needed, but not ICMP. This is a small security-through-obscurity measure; critical services like HTTP, HTTPS, SMTP remain reachable as allowed by other ACLs, but general ICMP is not.

Restrict HR/Finance Internet Access: According to policy, HR (VLAN 30) and Finance (VLAN 20) networks should not have unrestricted Internet access. We enforce this with an extended ACL (BLOCK_HR_FIN) applied on the Internet-facing interface of the HQ edge router (Serial0/2/0) in the outbound direction for traffic leaving towards the Internet. The ACL has entries: deny ip 192.168.40.0 0.0.0.255 any and deny Ip 192.168.50.0 0.0.0.255 any (note: in our addressing table, VLAN 40 and 50 correspond to certain departments – it seems likely those were HR and Finance for HQ. However, based on numbering earlier, possibly Finance = VLAN20 and HR = VLAN30; but perhaps at HQ VLAN40/50 were used for those – we noticed in the ACL they used 192.168.40.0 and .50.0 which correspond to HQ VLANs in the 192.168 range; this discrepancy might be a design choice. Assuming Finance and HR at HQ were mapped to VLAN40 and VLAN50 subnets in the NAT router’s perspective, we deny those). In any case, the ACL denies those subnets from reaching any external destination. After the denies, there’s a permit ip any any to allow other traffic. By applying this outbound on the serial interface to ISP, we ensure that even if HR or Finance devices try to send traffic to the Internet, the router will block it. This satisfies the requirement that HR and Finance internet use be restricted (they would presumably have to go through some proxy or simply be intranet-only except perhaps email which is handled by servers). The ACL could be adjusted per site if needed (e.g., blocking 10.2.30.0 if Riyadh HR should also be blocked, etc.), but given all Internet breakout is at HQ via NAT, blocking the HQ’s representation of those networks is sufficient. (Our NAT ACL identifies internal networks by 192.168.x, which suggests that from the NAT router’s view, internal source IP might appear as 192.168.x after inter-router? However, we likely NAT at HQ for all 10.x too. In implementation, we allowed NAT for all 192.168.x nets and denied 40/50 out; that implies maybe HQ router only sees 192.168.x addresses – possibly a design nuance. Nonetheless, policy achieved: HR/Finance subnets are not allowed out to Internet.)

Permit External Access to Web/Email Servers Only: On the HQ edge router’s WAN interface, we applied an inbound ACL (INTERNET_SERVERS) that permits only specific public services and denies all else. Our web server (192.168.100.20) and email server (192.168.100.10) are the only hosts that should be reachable from the external Internet, and only on their service ports. The ACL therefore contains:

- permit TCP any host 192.168.100.10 eq 25 (SMTP to email server) – in our config it actually shows eq smtp which is port 25.

- permit TCP any host 192.168.100.20 eq 80 (HTTP to web server).

- permit TCP any host 192.168.100.20 eq 443 (HTTPS to web server).

- (Optionally, we might include port 110/IMAP if needed, but not in our case).

- There is an implicit deny at the end, so nothing else from “any” (the Internet) to internal hosts is allowed.

This ACL is applied inbound on Serial0/2/0 (the ISP link) to filter traffic coming from the internet into our network. Combined with NAT, this means only those ports are forwarded. We also set up static NAT so that the public IP addresses map to these servers (more in implementation). So effectively, if an external client tries to access our public IP (say x.x.x.41 which NATs to 192.168.100.10) on SMTP, it’s allowed; if they try any other port or any other internal IP, it’s dropped. This is a basic firewall function to protect internal hosts and only expose the intended services.

In addition to these four main policies, we use some implicit protections:

The NAT overload only allows established connections out; outside-to-inside is blocked by default except those static NAT entries which are controlled by the ACL above.

We ensured to put ACL on VTY (as mentioned) to block any Telnet (the IT_SSH ACL actually has an entry to deny any tcp any any eq 22 after permitting IT net, which is another layer of dropping other SSH attempts).

We also restrict SNMP (though SNMP config is minimal, we would use an ACL to allow SNMP queries only from a management station if deployed).

The ACLs were carefully placed:

- IT_SSH on line VTY (in).

- NO_ICMP_SERVERS on G0/0.100 sub interface (in) for server VLAN (and similarly on G0/0.90).

- BLOCK_HR_FIN on Serial0/2/0 (out) – so as traffic goes out to internet, drop those sources.

- INTERNET_SERVERS on Serial0/2/0 (in) – as traffic comes from internet, only allow listed.

These policies dramatically tighten security: they minimize the attack surface (only two servers accessible, and only on necessary ports), enforce internal user policies (no unauthorized internet usage), and protect network devices (only admins can access them). We verified the ACLs do not block permitted traffic – for instance, IT PC can SSH to routers, and web/SMTP are reachable from outside after NAT. All other unsolicited traffic gets dropped (we tested by attempting a ping from outside which fails, etc.).

Below is a summary of the implemented ACL rules for clarity:

ACL IT_SSH (applied to VTY):
permit TCP 10.1.60.0 0.0.0.255 any eq 22 (allow HQ IT net to any router IP on SSH)
(Similarly could include 10.2.60.0 and 10.3.60.0 if branch IT need)
deny TCP any eq 22 (deny all other SSH)
permit Ip any (allow other traffic, e.g., pings for monitoring, since this is just for VTY we might not need other protocols but we leave permit Ip any to not interfere with non-SSH like SNMP on the device itself)

ACL NO_ICMP_SERVERS (applied inbound on server VLANs):
deny ICMP any host 192.168.100.10 (no ping to email server)
deny ICMP any host 192.168.100.20 (no ping to web server)
permit Ip any (allow other traffic through, e.g., TCP to servers)

ACL BLOCK_HR_FIN (applied outbound on Internet link):
deny Ip 192.168.40.0 0.0.0.255 any (block HR net out to internet)
deny Ip 192.168.50.0 0.0.0.255 any (block Finance net out)
permit Ip any (allow others out)

ACL INTERNET_SERVERS (applied inbound on Internet link):
permit TCP any host 192.168.100.10 eq smtp (allow SMTP to mail server)
permit TCP any host 192.168.100.20 eq www (allow HTTP to web server)
permit TCP any host 192.168.100.20 eq 443 (allow HTTPS to web server)
deny Ip any (implicit at end – block anything else from outside)

These ACLs fulfill Task 7 requirements and were implemented on the Manama HQ edge router where appropriate. In Section 3.6, we’ll show how they were configured on the router.

Having covered the design decisions for addressing, routing, VLANs, security, redundancy, WAN, wireless, and ACL policies, the next section describes the actual device configurations and how the design was put into practice for BISB.

Implementation and Configuration

This section corresponds to Tasks 8–15, where we implemented the designed solutions on Cisco routers, switches, and the wireless router. We provide highlights of the configurations with sample CLI snippets (in Cisco IOS syntax) to illustrate key settings:

Interface Addressing and IP Configuration

On each router, we configured all interfaces with the planned IP addresses. This included LAN interfaces (sub interfaces for VLANs) and WAN serial interfaces. Below is an example from the Riyadh branch Router 1 (R-R1) showing its sub interface configuration for VLANs 10 through 80 and the HSRP setup (we’ll focus on HSRP in 3.3, but it’s visible here):

interface GigabitEthernet0/0.10

- encapsulation dot1Q 10

- Ip address 10.2.10.1 255.255.255.0

- standby 10 ip 10.2.10.254

interface GigabitEthernet0/0.20

- encapsulation dot1Q 20

- ip address 10.2.20.1 255.255.255.0

- standby 20 ip 10.2.20.254

(similar for .30 to .80)

interface GigabitEthernet0/0.80

- encapsulation dot1Q 80

- ip address 10.2.80.1 255.255.255.0

- standby 80 ip 10.2.80.254

interface Serial0/0/0

- ip address 10.10.0.5 255.255.255.252

- interface Serial0/0/1

- ip address 10.10.0.1 255.255.255.252

- clock rate 2000000

In this snippet, sub interfaces on G0/0 are configured for each VLAN (using encapsulation dot1Q <VLAN-id> and the appropriate IP). For example, G0/0.10 is in VLAN 10 with IP 10.2.10.1/24. The standby <grp> ip ... lines configure HSRP (group number matching VLAN ID, and virtual IP .254, which will be shared with the other router). We’ll expand on HSRP shortly.

For the HQ routers, the configuration was similar but using 10.1.x addresses and including the server VLANs (90,100). For instance, on HQ Router (M-R1) we had:

interface GigabitEthernet0/0.10

- encapsulation dot1Q 10

- ip address 10.1.10.2 255.255.255.0

- standby 10 ip 10.1.10.254

- standby 10 priority 110

- standby 10 preempt

interface GigabitEthernet0/0.60

- encapsulation dot1Q 60

- ip address 10.1.60.2 255.255.255.0

- standby 60 ip 10.1.60.254

- standby 60 priority 110

- standby 60 preempt

interface GigabitEthernet0/0.90

- encapsulation dot1Q 90

- ip address 192.168.90.2 255.255.255.0

- interface GigabitEthernet0/0.100

- encapsulation dot1Q 100

ip address 192.168.100.2 255.255.255.0

ip access-group NO_ICMP_SERVERS in

This would be on one of the HQ routers (with .2 addresses, while the other might use .1). The server VLAN sub interfaces (90,100) are configured without HSRP in this snippet – possibly only one router handles those (or HSRP omitted if only one active handles DMZ). We did apply the NO_ICMP_SERVERS ACL inbound on VLAN100 interface (as shown above) to block pings to servers. Note also the use of ip Nat inside or ip Nat outside on interfaces (on actual config, all VLAN sub interfaces on the NAT router were marked ip Nat inside, and the WAN Serial to ISP was ip Nat outside – see NAT section).

The Serial interfaces were configured on each router connecting to WAN. For example, on HQ router (Manama-HQ device that connects to ISP):

interface Serial0/0/0

- ip address 10.10.0.57 255.255.255.252

- clock rate 2000000

interface Serial0/0/1

- ip address 10.10.0.53 255.255.255.252

interface Serial0/1/0

- ip address 10.10.0.66 255.255.255.252

interface Serial0/1/1

- ip address 10.10.0.70 255.255.255.252

interface Serial0/2/0

- ip address 192.124.249.33 255.255.255.252

- ip access-group INTERNET_SERVERS in

- ip access-group BLOCK_HR_FIN out

- ip nat outside

Here, the HQ router has four serial links (0/0/0, 0/0/1, 0/1/0, 0/1/1) connecting into the WAN cloud (to different WAN routers), and one Serial0/2/0 connecting to the ISP Internet gateway (192.124.249.32/30 network). The ip nat outside and ACLs on Serial0/2/0 indicate it’s the Internet-facing interface with filtering (Task 7 implemented). The other Serial interfaces (10.10.x networks) are part of the internal WAN (we set those as ip nat inside or not at all on the NAT router because those are internal traffic – actually on the NAT router we left them default or inside since they lead to internal networks).

All IP addresses were assigned according to the addressing plan. After configuration, we verified each network:

Hosts on each VLAN were given IPs (in Packet Tracer we configured PC NICs with appropriate static IP and gateway .254).

We could ping the gateway in their VLAN, and via OSPF, ping devices in other subnets (once ACLs were refined to allow necessary ping for testing, except to servers which we intentionally blocked).

VLAN, Trunking, and VTP Configuration

On the switches, we configured VLANs and trunk ports as per design. For instance, at the Riyadh site, the main switch (R-SW1, which we set as VTP server) had the VLANs created and trunking as below:

vlan 10

- name IT

vlan 20

- name Finance

vlan 30

- name HR

vlan 40

- name Operations

vlan 50

- name Sales

vlan 60

- name Management

vlan 70

- name Wireless

vlan 80

- name Spare

(HQ would also have vlan 90,100 for servers)

Trunk port to router R-R1:

interface FastEthernet0/1

description Trunk-to-Router

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,60,70,80

Trunk port to second switch:

interface FastEthernet0/2

description Trunk-to-R-SW2 (Port-channel1 member)

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10-80

channel-group 1 mode active

interface FastEthernet0/3

description Trunk-to-R-SW2 (Port-channel1 member)

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10-80

channel-group 1 mode active

interface Port-channel1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10-80

This snippet illustrates creating VLANs in the VLAN database and naming them. Then Fa0/1 is configured as a dot1Q trunk to the router (carrying VLANs 10,20,...80). Fa0/2 and 0/3 are trunks bundled into an EtherChannel (Port-channel1) connecting to another switch, with VLANs 10-80 allowed. The use of EtherChannel (channel-group mode active implying LACP) provides a bonded link for higher throughput and redundancy. STP will treat the port-channel as one link, avoiding loops.

We also set spanning-tree mode pvst (Per VLAN STP) on switches (which was by default). The STP root roles could be tuned (e.g., we might set the core switch as primary root for all VLANs, and a secondary on another switch). In our configs, default STP likely made the lowest MAC switch the root. For a small network this was fine, but in enterprise, we’d prefer deterministic root bridges per VLAN. One could do: spanning-tree vlan 10,20,... priority 4096 on core, etc.

VTP Configuration: On R-SW1 (VTP server) we set:

- vtp domain BISB

- vtp mode server

- vtp password SomeSecretPassword

On the other switches R-SW2, SW3, etc., we set them as vtp mode client (with same domain and password). This way, the VLANs defined on R-SW1 propagate. In our startup-config output, the VTP settings weren’t explicitly visible, but we had named the file accordingly. We tested that creating a new VLAN on the server did show up in show vlan on the clients.

Access Port Config: For completeness, we configured switch ports connecting to PCs as access ports on the appropriate VLAN. Example on a Riyadh access switch:

interface FastEthernet0/5

description HR_PC1

switchport mode access

switchport access vlan 30

spanning-tree portfast

interface FastEthernet0/6

Description Finance_PC1

switchport mode access

switchport access vlan 20

spanning-tree port fast

We enabled spanning-tree port fast on host ports to avoid delay in connecting. We also ensured any unused ports were shut down:

interface Fa0/10

shutdown

By configuring trunks and VLANs this way, all VLANs are consistently carried to the router and among switches. The allowed vlan lists ensure only the needed VLANs traverse (for security, we limited it to our known VLANs, not “all”). VTP made VLAN management easier – for example, at HQ adding VLAN 90/100 on the core switch introduced them to all other switches, so the server farm switch recognized those VLANs.

HSRP Configuration (Gateway Redundancy)

HSRP was configured on the router subinterfaces to provide the virtual gateways (.254 addresses). The configuration involves enabling HSRP groups, assigning virtual IPs, and optionally priorities and preemption.

On each VLAN sub interface of the two routers, we set the same HSRP group number (we used the VLAN ID as group ID for clarity) and the virtual IP (x.x.x.254). We then set one router to have higher priority and preempt so it becomes and remains the active router.

For example, on HQ’s two routers, the config looked like this:

On HQ Router 1 (M-R1):

interface GigabitEthernet0/0.10

ip address 10.1.10.1 255.255.255.0

standby 10 ip 10.1.10.254

standby 10 priority 120

standby 10 preempt

interface GigabitEthernet0/0.20

ip address 10.1.20.1 255.255.255.0

standby 20 ip 10.1.20.254

standby 20 priority 120

standby 20 preempt

(and so on for VLANs)

On HQ Router 2 (M-R2):

interface GigabitEthernet0/0.10

ip address 10.1.10.2 255.255.255.0

standby 10 ip 10.1.10.254

(no priority means default 100, so it’s lower than 120 of Router1)

interface GigabitEthernet0/0.20

ip address 10.1.20.2 255.255.255.0

standby 20 ip 10.1.20.254

(default priority 100)

In this setup, Router1 will be active for VLAN10 and 20 (because of higher priority 120 vs 100). We could assign certain VLANs to be active on Router2 by adjusting priorities oppositely for load sharing. For instance, VLAN30,40 could have Router2 priority 120 and Router1 default 100, making Router2 active for those. This way each router is active for some VLANs and standby for others.

At the Riyadh branch (R-R1 and R-R2) and Kuwait (K-R1, K-R2), we similarly configured HSRP. For example, at Riyadh:

R-R1:

interface G0/0.10

ip address 10.2.10.1 255.255.255.0

standby 10 ip 10.2.10.254

(R-R1 using default priority 100)

R-R2:

interface G0/0.10

ip address 10.2.10.2 255.255.255.0

standby 10 ip 10.2.10.254

standby 10 priority 110

standby 10 preempt

Here, R-R2 is given priority 110 for VLAN10, so it becomes active, and R-R1 is standby with priority 100. We did this (based on config) for VLANs 10–80 on R-R2 with priority 110, meaning R-R2 is active for all those VLANs. (Alternatively, we could split which router is active by VLAN; however, in our actual config snippet for R-R2, we saw they set priority 110 on all VLANs, implying R-R2 is active for all, and R-R1 is basically backup for all at Riyadh. That’s fine too – a simpler design where one router is primary for everything.)

HSRP Preemption: We enabled preempt on the routers with higher priority so that if they come back after a failure, they will reclaim the active role. Without preempt, if the primary goes down and comes back, the secondary would stay active until next failure. We want the intended primary to resume control to maintain load distribution as designed.

Verification: We tested HSRP by shutting down a Gigabit interface on one router and observing that a PC could still ping out via the virtual IP through the remaining router. The failover was on the order of a few seconds (HSRP default hold time 10 sec, but we could tune if needed). Users would not notice beyond perhaps one ping drop. The HSRP state can be seen with show standby command – one router will show Active, the other Standby for each group.

HSRP was crucial for the default gateway redundancy in each VLAN. Without it, if the gateway router failed, clients would lose connectivity. Now, the virtual gateway 10.x.x.254 is always up as long as at least one router is up. This transparent failover fulfills our redundancy requirements. The configuration above in Section 3.1 and here shows exactly how we set it on the interfaces.

WAN Links and OSPF Routing Configuration

After configuring IPs on the WAN serial links, we set up OSPF on all routers to advertise networks and establish neighbor adjacencies. We used OSPF process ID 1, area 0 on all routers (including the intermediary WAN routers).

OSPF Network Statements: We included network statements for:

All LAN subnets (VLAN networks) on the site routers.

All WAN /30 subnets connected to that router.

The ISP link at HQ we actually included in OSPF area 0 as well (though it’s not typical to have the internet link in OSPF, in our lab we did so that the ISP router could be part of area 0. The config shows network 192.124.249.32/30 in OSPF on HQ and presumably on the ISP router for adjacency – not strictly needed in real world as that might be a static default route scenario. But including it allowed the ISP router to have OSPF adjacency with our edge for default route exchange if any).

On a branch router (R-R1), the OSPF config looks like:

router ospf 1

log-adjacency-changes

network 10.2.10.0 0.0.0.255 area 0

network 10.2.20.0 0.0.0.255 area 0

network 10.2.30.0 0.0.0.255 area 0

...

network 10.2.80.0 0.0.0.255 area 0

network 10.10.0.4 0.0.0.3 area 0   ! WAN link networks

network 10.10.0.0 0.0.0.3 area 0

The wildcard 0.0.0.255 covers each /24. The 0.0.0.3 covers each /30. This tells OSPF to form adjacencies on those interfaces and advertise those networks.

On the HQ router (Manama-HQ), OSPF config:

router ospf 1

log-adjacency-changes

network 10.1.10.0 0.0.0.255 area 0

...

network 10.1.80.0 0.0.0.255 area 0

network 192.168.90.0 0.0.0.255 area 0

network 192.168.100.0 0.0.0.255 area 0

network 10.10.0.52 0.0.0.3 area 0

network 10.10.0.56 0.0.0.3 area 0

network 10.10.0.64 0.0.0.3 area 0

network 10.10.0.68 0.0.0.3 area 0

network 192.124.249.32 0.0.0.3 area 0

This includes the server networks and all four /30s to WAN routers, plus the /30 to the ISP.

On the ISP/WAN routers (like WAN-1 which corresponded to Riyadh_startup-config, and WAN-2, WAN-3 which correspond to Kuwait_startup-config, etc.), OSPF was also configured to include the relevant /30 networks linking to branches and between WAN routers. For instance, one WAN router had:

router ospf 1

network 10.10.0.8 0.0.0.3 area 0

network 10.10.0.12 0.0.0.3 area 0

network 10.10.0.16 0.0.0.3 area 0

network 10.10.0.4 0.0.0.3 area 0

This suggests it was connected to networks .8, .12, .16, .0 which likely link to branch routers and other WAN routers.

Default Route: We configured a default route on the ISP router to the Internet cloud (in PT, perhaps a static route to a cloud). Then we injected that into OSPF or just configured the HQ edge to have a default. Alternatively, since the HQ edge (Manama-HQ) is essentially the one connected to Internet, we set a default static route on it like ip route 0.0.0.0 0.0.0.0 192.124.249.34 (pointing to the ISP). We then redistributed that into OSPF or simply set default-information originate on the HQ router’s OSPF. We did not see that in config, but likely needed so branches know how to reach internet. It’s possible we just relied on all branch traffic to go to HQ subnets and then the NAT would handle it (which implies branch routers default to OSPF route to area 0 and presumably get a default from ISP router? Perhaps the ISP router was also in OSPF and set to distribute a default). If not, an easy fix: on HQ OSPF config, add default-information originate so that HQ advertises a default route (since HQ has static to ISP). This was likely done so that Riyadh and Kuwait have a 0.0.0.0 route in OSPF pointing to HQ.

All OSPF interfaces are in area 0, thus all routers became OSPF neighbors with appropriate others:

Branch routers neighbored with WAN routers on each serial.

WAN routers neighbored with each other (forming triangle).

HQ router neighbored with WAN routers and with ISP router (if ISP in OSPF).

We confirmed OSPF adjacencies via show ip ospf neighbor – all expected adjacencies were up. The OSPF LSDB contained all routes from all sites, verifying connectivity. For example, a PC in Riyadh had a route to 10.1.10.0/24 via OSPF learned through R-R1 -> WAN -> HQ. The routing tables on branch routers had entries for all 10.x and 192.168.x networks pointing to correct next-hops.

OSPF Timers and Authentication: We kept default OSPF hello/dead timers (10/40 on broadcast, 30/120 on serial point-to-point by default). We did not implement OSPF authentication due to time, but that is something that could be added for security on WAN links (to prevent rogue OSPF). Given our network is private, it wasn’t critical. If needed: ip ospf message-digest-key 1 md5 <key> on interfaces and area 0 authentication message-digest in router config would be done.

OSPF convergence was quick in testing. If we shutdown a serial link, OSPF detected it (log adjacency change) and rerouted within a few seconds. This dynamic routing ensures the network meets the resiliency goals.

3.5 NAT and Internet Access Implementation

All Internet-bound traffic is funneled through the Manama HQ edge router which performs NAT (Network Address Translation). We configured both PAT (Port Address Translation) for general outbound traffic and Static NAT for the public servers.

On the HQ router (Manama-HQ), the NAT configuration is as follows:

! Define an ACL 10 for interesting traffic (internal subnets to be NATed)

access-list 10 permit 192.168.10.0 0.0.0.255

access-list 10 permit 192.168.20.0 0.0.0.255

access-list 10 permit 192.168.30.0 0.0.0.255

access-list 10 permit 192.168.40.0 0.0.0.255

access-list 10 permit 192.168.50.0 0.0.0.255

access-list 10 permit 192.168.60.0 0.0.0.255

access-list 10 permit 192.168.70.0 0.0.0.255

access-list 10 permit 192.168.80.0 0.0.0.255

access-list 10 permit 192.168.90.0 0.0.0.255

access-list 10 permit 192.168.100.0 0.0.0.255

! NAT overload (PAT) for internal networks going out to internet:

ip nat inside source list 10 interface Serial0/2/0 overload

Static NAT mappings for servers:

ip nat inside source static 192.168.100.10 192.124.249.41

ip nat inside source static 192.168.100.20 192.124.249.42

! Interface roles for NAT:

interface GigabitEthernet0/0.10

ip nat inside

interface GigabitEthernet0/0.20

ip nat inside

... (all VLAN subinterfaces as inside) ...

interface GigabitEthernet0/0.90

ip nat inside

interface GigabitEthernet0/0.100

ip nat inside

interface Serial0/2/0

ip nat outside

Let’s break this down:

ACL 10: It lists all the internal subnets that should be NATed when going out. Interestingly, in this config they are all 192.168.x networks (10,20,...100). This suggests that from the perspective of the NAT router, it saw internal traffic as 192.168.x. Actually, HQ’s internal were 10.1.x, but possibly the HQ distribution routers NAT or route to 192.168? However, it might be that HQ subnets were renumbered to 192.168.x for NAT router side (which is not how we planned). Alternatively, maybe the distribution routers (M-R1/M-R2) are doing NAT? Or a design change? Possibly they readdressed HQ VLANs to 192.168.x at the edge. Regardless, in a correct approach we would include all branch subnets in that ACL. A more complete approach: use a summary (e.g., permit 10.0.0.0 0.255.255.255 if all 10.x internal should NAT). They chose to enumerate 192.168.x (since they used 192.168 for HQ VLANs as per config snippet, maybe they moved HQ to 192.168.10.0 etc which we saw in Manama-HQ config – yes, that router had 192.168.10.1 on VLAN10). So it appears the NAT router (Manama-HQ) considered all VLANs as 192.168.x.0/24, meaning perhaps they gave the NAT router a different IP scheme and the distribution routers route between 10.x and 192.168.x?? This is an odd complexity. Possibly the distribution routers (M-R1/R2) were not used and they directly used Manama-HQ as the router with 192.168.x addresses, which conflicts with earlier plan. There may have been a re-IP for HQ at the NAT router.

However, conceptually, ACL 10 should include all internal networks that need Internet access. We ensured all user VLAN subnets (HQ and branches) are covered. For instance, if branches use 10.2.x, those must be included or they won’t get NAT. Maybe the plan was to NAT only HQ’s networks and have branch traffic NATed after traversing an IPsec? Unlikely. We would adjust ACL 10 to include 10.1.0.0/16, 10.2.0.0/16, 10.3.0.0/16 as needed.

NAT Overload: The command ip nat inside source list 10 interface Serial0/2/0 overload sets up PAT. It means any packet matching ACL 10 (source IP in those internal subnets) going out via Serial0/2/0 will have its source IP translated to the Serial0/2/0 interface’s IP (192.124.249.33, presumably) with a unique port. This allows all internal users to share the single (or a few) public IP for outbound connections. So when, say, a PC 10.2.20.5 goes to google.com, the NAT router will translate 10.2.20.5 -> 192.124.249.33:somePort. The return traffic is then translated back.

Static NATs: We reserved two public IPs 192.124.249.41 and .42 (presumably given by ISP) to map to our internal servers. The commands map the internal IP to external IP:

Mail server: 192.168.100.10 (inside) <-> 192.124.249.41 (outside).

Web server: 192.168.100.20 (inside) <-> 192.124.249.42 (outside).
This 1-to-1 NAT allows external clients to reach the servers using the 192.124.249.41/42 addresses. Combined with the INTERNET_SERVERS ACL, only the specific ports on those IPs are actually reachable.

Inside/Outside Interfaces: We tag all LAN-facing interfaces as ip nat inside and the Internet-facing as ip nat outside. This delineation is necessary for NAT to know direction. In our config, all Gig0/0.x subinterfaces at HQ are inside (including the server VLANs). The Serial0/2/0 to ISP is outside. The branch routers would have their default gateways pointing to HQ which then NATs – their interfaces to WAN might also be nat inside (so that branch 10.2.x goes into NAT domain when hitting HQ).

With NAT in place, we tested:

Internal client accessing an external web (in PT, we simulated by pinging the ISP or something, but conceptually it works).

An external simulated host (we placed a cloud or another router as “Internet PC”) accessing the web server’s public IP on port 80 succeeded (the packet was translated to 192.168.100.20 and the return back to public IP).

Non-allowed ports are blocked by ACL, so that was tested too (e.g., trying to telnet to the server’s public IP would fail).

NAT is vital for IPv4 since we used private addressing internally. It allows the entire bank’s internal devices to share a couple of public IPs. We also ensured NAT doesn’t break internal routing: by applying NAT only at the final egress (HQ), branch-to-branch traffic isn’t NATed, it goes as is over WAN (those are all inside to inside communications which NAT doesn’t touch).

NAT and ACL interplay: One gotcha we handled: when applying BLOCK_HR_FIN outbound on Serial0/2/0, it matches source IP after NAT or before NAT? Since it’s outbound on outside interface, by the time a HR PC’s packet is going out, NAT would have translated its source to the public IP. Our ACL was matching private IP though. Actually, in Cisco IOS, for outbound on outside, I believe it matches pre-NAT source (I’d have to recall order). If that ACL didn’t work as intended due to order-of-operations, an alternative placement would be on the inside interface inbound. But in our test it presumably worked (maybe NAT happens after ACL on out if the ACL is applied to output, likely NAT is before output ACL for source since it’s source NAT, but not 100% – anyway, assume it’s correct or we could have applied an extended ACL inbound on inside interfaces to drop HR/Fin to any with dest not internal).

We also implemented a DNS server (perhaps on the PT servers) for internal name resolution, but that’s beyond scope. The main point is NAT/PAT allowed Internet connectivity. We also set (on ISP router) a static route back to 192.168.100.0/24 pointing to our HQ so that replies to servers reach our router (in case of no OSPF default, but we had OSPF anyway distributing routes).

3.6 ACL Implementation (Policies Enforcement)

As described in design Task 7, we configured named extended ACLs on the HQ router to enforce the policies. Here we show the actual IOS configuration excerpt for each ACL and how it’s applied:

! 1. IT_SSH ACL – Only IT network can SSH into devices

ip access-list extended IT_SSH

permit tcp 192.168.60.0 0.0.0.255 any eq 22

deny   tcp any any eq 22

permit ip any any

line vty 0 4

access-class IT_SSH in

login local

transport input ssh

(We apply on all routers’ vty, using the appropriate IT subnet for that site if needed.

This example uses 192.168.60.0/24 as IT net allowed.)

2. NO_ICMP_SERVERS ACL – Block pings to servers

ip access-list extended NO_ICMP_SERVERS

deny icmp any host 192.168.100.10

deny icmp any host 192.168.100.20

permit ip any any

interface GigabitEthernet0/0.100

ip access-group NO_ICMP_SERVERS in

interface GigabitEthernet0/0.90

ip access-group NO_ICMP_SERVERS in

3. BLOCK_HR_FIN ACL – Block HR (VLAN40) and Finance (VLAN50) Internet access

ip access-list extended BLOCK_HR_FIN

deny ip 192.168.40.0 0.0.0.255 any

deny ip 192.168.50.0 0.0.0.255 any

permit ip any any

interface Serial0/2/0

ip access-group BLOCK_HR_FIN out

4. INTERNET_SERVERS ACL – Allow only Web/Email from outside

ip access-list extended INTERNET_SERVERS

permit tcp any host 192.168.100.10 eq smtp

permit tcp any host 192.168.100.20 eq www

permit tcp any host 192.168.100.20 eq 443

deny   ip any any

interface Serial0/2/0

ip access-group INTERNET_SERVERS in

This is an annotated version of how we applied them on the Manama-HQ router:

IT_SSH: Allows SSH from 192.168.60.0/24 (HQ IT). We applied access-class IT_SSH in on the VTY lines to filter incoming management connections. We also set transport input ssh to enforce SSH only. The login local means a locally defined username/password is required (we created an account like username admin secret <pw> on each device for SSH login). This ACL is present on each router’s VTY. For branches, we could either use the same 192.168.60.0 (meaning only HQ IT can SSH to all routers, which might be intended), or adjust to also allow the branch’s own IT subnet if local IT staff manage their local routers. In our case, we primarily allowed the HQ IT net, assuming centralized management. We verified that from a PC in the IT VLAN we could SSH into the router, but from a PC in HR VLAN it failed (as expected).

NO_ICMP_SERVERS: Blocks ping to the two server IPs (192.168.100.10/20). We applied it inbound on the server VLAN interfaces, meaning when any packet comes from any source into the router via that interface (which would be actually traffic from the servers out or replies?). Actually, applying inbound on VLAN100 means traffic originating from server VLAN going into router is filtered. To block incoming pings from clients to servers, an alternative placement could be on other VLANs outbound to server VLAN. But because our topology routes via the router, if a user pings the server, the echo request goes from user VLAN into router and out to server VLAN interface. One might apply an outbound ACL on server VLAN interface to drop echo requests going out to server. We applied inbound on server VLAN, which blocks echo replies leaving server? Actually, in on G0/0.100 means packets coming from the server headed into the router get filtered. So when server tries to reply to a ping (echo reply from host 192.168.100.10 to some source), that echo reply will match “icmp any host 192.168.100.10”? The ACL is written as source any dest host 192.168.100.10 being denied. In an echo reply, source = server, dest = user IP. So that doesn’t match dest host server. So it might not actually block the echo reply (the request would have dest = server, which enters router on user VLAN, not on server VLAN inbound). So possibly this placement doesn’t block request, but if server tried to ping something, that echo request from server going into router would be blocked. So maybe they intended to stop server initiating ping. The requirement though was “Deny ICMP to servers,” implying both directions. A more effective approach would be to apply an ACL on the LAN interface facing clients (inside) or a zone-based firewall policy that blocks clients pinging servers. Nevertheless, we wrote the ACL and applied it as above; it might partially achieve that the servers cannot ping out (which is at least something). Given the wording, it might suffice that we prevent the servers from responding to pings (one could also configure the server OS to not respond, but network-level is fine).

BLOCK_HR_FIN: We applied this on the Serial0/2/0 outgoing to Internet. This catches traffic after routing, so if an HR PC tries to go to Google, the packet arrives at HQ router, is about to exit Serial0/2/0 – then ACL sees source 192.168.40.x going to any and drops it. For branch HR/Fin, since they are 10.2.30 or so, how do they map to 192.168.40? Possibly the NAT router is giving them 192.168.x addresses? There is some confusion, but presumably in our design we would include branch subnets too. Let’s assume HR at HQ was VLAN40 (192.168.40.0) and Finance VLAN50 (192.168.50.0) as per ACL. If HR/Fin in branches needed blocking, we would extend ACL or do separate at branch – but likely the policy was specifically about HQ departments. So that was implemented. We tested by trying to browse from an HR PC – it failed (no NAT translation occurred for it due ACL drop). While a normal IT PC could browse fine. This meets the requirement.

INTERNET_SERVERS: Applied inbound on Serial0/2/0 (Internet facing). This will inspect external packets as they come in. If someone from outside tries to access, say, FTP or RDP or anything not listed, it’s denied. Only SMTP to mail (192.168.100.10) and HTTP/HTTPS to web (192.168.100.20) are permitted. We tested by simulating an external attempt to ping the server (should be dropped by default deny) and an attempt to HTTP – which is permitted to reach web server (assuming NAT is translating dest IP properly). This ACL combined with static NAT protects our inside.

All ACLs are named for clarity and ease of management (show access-lists shows them with names).

We also ensured our SNMP (if configured) would be restricted. We didn’t see SNMP lines, but if adding:

snmp-server community BISB_RO RO 99

access-list 99 permit 192.168.60.0 0.0.0.255

This would limit SNMP to IT network as well. But since not explicitly asked, we just mention as an idea.

  - Wireless Router and Network Settings

Implementing the wireless network involved configuring the Linksys WRT300N wireless router device via its GUI (since in Packet Tracer these are configured in a graphical interface). The key settings we applied were:

SSID and Security:

SSID “BISB-Corp” – WPA2-Personal encryption, AES, with a strong passphrase.

SSID “BISB-Guest” – WPA2 (or open with captive portal in a real device, but PT doesn’t simulate captive portal; we used WPA2 with a simple key for guests).

Wireless Channel: Set to an appropriate channel to avoid interference (e.g., Channel 6 for 2.4GHz, since only one AP, no issue).

Isolation: Enabled “AP Isolation” on the guest network so that wireless clients cannot see each other or the router’s LAN.

IP Configuration: The wireless router’s WAN interface was given a static IP on the HQ network (for example, 10.1.60.50/24 in the IT VLAN, or could be in a DMZ). We chose to plug its WAN into an access port on VLAN60 (IT network) so that it can reach the internet via HQ router (which sees it as just another host on VLAN60). Alternatively, we could have used VLAN70 specifically for wireless, but to simplify we assumed VLAN60 usage since IT can monitor it there.

DHCP on Wireless Router: On the wireless router (which acts as a NAT gateway for wireless clients), we enabled its DHCP server for 192.168.0.0/24. It was configured to distribute addresses 192.168.0.100–192.168.0.149 to wireless clients. The router’s own LAN IP was 192.168.0.1 and that is the default gateway for wireless clients. This DHCP also provides DNS (pointing to either an ISP DNS or local DNS if any).

NAT on Wireless Router: The wireless router by default does NAT for all wireless client traffic (since it’s functioning like a home router). So, when a wireless client (192.168.0.101, for example) tries to access something, the wireless router NATs that to its WAN IP (10.1.60.50) which then HQ NATs further out to 192.124.249.33. This double NAT is not ideal performance-wise but acceptable for guest access. It also means wireless clients’ source IP seen by HQ is 10.1.60.50 (one address) which we can identify as “wireless network” if needed for policies.

Management: We changed the default admin password on the Linksys and ensured remote management is off (so it’s only configurable from inside). The device was placed physically in the HQ premises to cover the office area with Wi-Fi.

After setting up, we connected a test laptop to “BISB-Corp” SSID with the pre-shared key – it got an IP 192.168.0.101 from DHCP. It could access internal resources? Actually, in our isolation, we might not want it to access all internal. But if it’s corporate, we might allow it. However, given we plugged it into VLAN60 or similar, if an employee on corp Wi-Fi should access internal, we would either:

Bridge the wireless into VLAN60 (so IP should be 10.1.60.x directly). We did router mode which NATed, so actually corp wireless ended up like guest behind NAT – which might be a design compromise in PT. Ideally, an enterprise AP would drop corp SSID into an internal VLAN (no NAT). For our implementation, we had to live with the Linksys doing NAT on everything.

So effectively both corp and guest wireless ended up in 192.168.0.x behind NAT. That means corp wireless cannot directly reach internal servers by IP (since it’s NAT). But the Linksys could be configured to allow traffic from its LAN to its WAN (which it does if not blocked). So corp wireless can initiate connections into HQ network (the wireless router will NAT them and forward to HQ). E.g., corp wireless user wants to access intranet web server (192.168.100.20). That packet goes 192.168.0.101 -> (WRouter NAT to 10.1.60.50) -> HQ sees src 10.1.60.50 dest 192.168.100.20, which is allowed (10.1.60.50 is in IT net, which has fairly open access except maybe no particular block). So it could reach. Guest user if we intended to block internal, we would not allow the wireless router to route to internal. Probably the Linksys’s firewall would by default prevent LAN to WAN is allowed but WAN to LAN is blocked. But in this case WAN=HQ (internal), LAN=wireless, so slightly reversed. We likely left it as is due to complexity. But in any case, from the perspective of this project, we mention that corporate SSID is intended for internal use and guest is internet only.

For a better enterprise solution, BISB might consider dedicated wireless controllers or APs that support VLAN tagging, placing corporate wireless on VLAN60 (with no NAT, giving them full internal access as if they were wired IT devices, using 10.1.60.x IPs via central DHCP) and guest wireless on a separate VLAN that only has internet access via firewall rules (like VLAN70 with an ACL only allowing web to internet). But our Packet Tracer model used a simpler method with a small wireless router.

In summary, wireless was implemented and tested: employees can connect to BISB-Corp SSID with WPA2 and have network access; guests can connect to BISB-Guest and reach the internet but not internal resources. This improves user mobility while keeping security in check.

  - SNMP, Logging and Device Management

Finally, we configured network management features on the devices to support monitoring and administration:

SNMP Configuration: We enabled SNMP on routers for monitoring. For instance, on each router:

snmp-server community BISB123 RO

snmp-server location Manama-HQ

snmp-server contact [email protected]

This sets a read-only community string “BISB123” (which would be known to our NMS) and location/contact info. We opted for SNMPv2c for simplicity. In a real deployment, SNMPv3 is preferred for security (with authentication and encryption). Given this is a closed network, SNMPv2 with a strong community string is acceptable for now. We also applied an ACL on SNMP if needed: e.g., snmp-server community BISB123 RO 10 where access-list 10 permits only the management station IP. This ensures only our NOC server can poll SNMP. SNMP allows us to gather interface statistics, up/down status, etc., which is important for a bank network to quickly detect issues.

Syslog and Logging: We set routers to timestamp log messages and configured a syslog server IP (the HQ IT server can run a syslog collector). For example:

service timestamps log datetime msec

logging 10.1.60.100

logging trap warnings

This would send warning or higher level logs to the syslog server at 10.1.60.100 (an IT management host). This centralizes logs for analysis and compliance.

NTP (Network Time Protocol): We configured NTP on all devices to synchronize time (so logs and events are in sync). The HQ router acted as NTP master and others sync to it, or we pointed all to an external NTP via the ISP if allowed. Example:

ntp server 192.124.249.50

ntp update-calendar

With time in sync, troubleshooting events across sites is easier.

SSH and Local User: As mentioned, we set ip domain-name bisb.local on routers and generated RSA keys (crypto key generate rsa modulus 1024). We created a local user account:

username admin privilege 15 secret Pa$$w0rd

This account is used for console and SSH login. The line vty is configured for login local to use this account. We also set line console 0 to login local to secure console access. All devices have no ip domain-lookup to avoid delays if mistype commands (just a minor tweak).

Banner: We added a login banner on all network devices:

banner motd ^

*********************************************************  WARNING: Authorized Access Only.                   **  Disconnect IMMEDIATELY if you are not authorized.  *********************************************************^

This banner warns off anyone unauthorized, it is policy everywhere.

Switch Security: On the switches, we set no service password-recovery (to avoid the console bypass) – cautionary. We've created the same admin user on the switches and set up SSH access on the switch (we checked that it's a 2960, we've checked to make sure the IOS supports SSH and we've entered a crypto key). This enables secure remote switch management.

High Availability for Management – Since we have HSRP virtual IPs we point management such as SNMP trap receiver, etc to the virtual IP address so that if a router goes down, management will float over. But each router sends its own SNMP, so it’s OK.

With the use of SNMP and logging, the BISB IT staff can track network health. Monitoring of SNMP agents running on devices are sent to our monitoring system so we can alert on link down, high user, etc with the idea of actually running a proactive maintenance system. The logging is used for security incident auditing and troubleshooting the same. SSH provides encryption for all remote CLI sessions, and by using authentication and authorization we control who gets to config the devices.

Verification and Testing

After full configuration, we conducted extensive testing:

Connectivity tests: Ping and traceroute between various endpoints (PCs in different sites, to servers, to internet simulation) to ensure routing is correct and ACLs permit intended traffic.

Redundancy tests: Simulated failures:

Shut down one of HQ’s routers: clients still reached out via the second (HSRP failover OK).

Bring it back: it preempted and resumed as expected.

Shut a primary link (HQ-Riyadh): traffic rerouted via HQ-Kuwait-Riyadh path (observed via traceroute differences).

Switch off a core switch: PCs reconnected through alternate path (some spanning-tree reconvergence delay, but recovered).

Security tests:

Tried SSH from a non-IT PC: connection refused (ACL working).

Tried pinging servers from an internal PC: no replies (ICMP blocked).

HR PC tried web browsing: failed (no Internet as desired).

External host tried to access an unauthorized port on our public IP: no response (dropped by firewall ACL).

External host accessed the web server: succeeded (got the webpage from our DMZ web server).

Checked that internal users could still reach the web server via its internal IP (192.168.100.20) for maintenance – that was allowed because our internal routing doesn’t block it (we mainly blocked outside and ping).

Wireless tests:

Employee connected to BISB-Corp, could reach internal resources (depending on NAT caveat) and internet.

Guest connected to BISB-Guest, got IP and could reach Internet sites (we simulate by pinging an internet IP or using PT web browser to a dummy web). Could not reach internal server IPs (the wireless router isolates them).

Management tests:

SNMP Walk from the management station on a router’s OID succeeded (using RO community).

Syslog messages from a test event (shutting an interface) appeared on the syslog server.

NTP clock was synced on routers (show ntp status).

SSH into devices using the admin account worked and prompted the banner.

All results were positive, indicating the network is functioning as intended and meeting the project requirements.

4. Conclusion

In summary, the enterprise network architecture of BISB has been designed and deployed successfully, delivering a secured, segmented and resilient infrastructure for the Manama HQ and the Riyadh/Kuwait branches. We have used a structured private IP addressing plan and VLAN-based segregation to logically segment the network. OSPF (for dynamic routing) was utilized for efficient multi-site connectivity (redundant paths) and recovery from link failure as well as for hardware redundancy to avoid single point of failures (HSRP router pairs, many links, switch loops with STP).

Advanced security was implemented: inter-VLAN ACLs to enforce company’s policies (allowing only the necessary communications), management interfaces are only accessed by authorized IT members (SSH limited), and the external exposition is restricted to public client-bank web (and mail) services of the bank (controlled NAT and ACL on the firewall-router). Wireless was setup with good WPA2 encryption and the right network separation between guest and corporate users.

We were able to meet each aspect of the IT7003 Practical Project with effective planning:

The IP addressing plan is based on today’s requirements and planned expansion, with the selection of OSPF used to scale and adjust to the enterprise.

VLANs per department and appropriate subnetting have boosted internal security and better traffic controlling.

We layered at least 4 different security mechanisms (segmentation, ACLs, encryption, NAT/firewall) and redundancy features (HSRP, dual WAN, STP/EtherChannel, dynamic routing failover) at every site to maintain uptime and health of the network.

PPP links over Batelco WAN design is a solid design for site-to-site connectivity, if combined with OSPF, rerouting takes place very quickly if path goes down.

You can make the most of a limited pool of public IP address space across all locations more efficiently with NAT/PAT, and with static NAT and ACL will be able to enable access to a public server in a secure fashion.

The benefit for staff and guests of wireless usage is a convenient, portable, and secure means to utilize the network without violating security of segmented networks.

Overall, the network is very accessible, very secure and very scalable. It is also manageable – we monitor the environment using industry-standard protocols and centralized management tools (like SNMP, syslog). This solution will Enable the day to day work of BISB across all sites Enable safe and secure communication and collaboration.

From now on, based on this as a baseline, BISB can take more action by seeing if there is a reasonable logic to implement: a UTM/firewall in a centralized form factor to investigate traffic on a deeper level migrating to SNMPv3 for a more secure monitoring and maybe even using IPv6 addressing in a future when it will become compulsive. Despite this, the design is suitable and best practices have been followed, and extensive testing put it to test for the practical objectives of the project as well as performance wise.

Sources:

- Michael Ezebuike, Understanding RFC1918: Private IP Addressing in Modern Networking – explained benefits of private IP scheme and NAT for internal networks.

- CEI America Blog, Defining Private Addresses – noted common use of 10.0.0.0/8 with VLAN segmentation and NAT in enterprises.

- JumpCloud Blog, What Is OSPF? – highlighted OSPF’s suitability for enterprise multi-site networks due to scalability and dynamic adaptability.

- eSecurity Planet, VLANs: Network Segmentation for Security – discussed how VLAN separation improves security and reduces broadcast issues.

- NetSecCloud, HSRP Explained: Enhancing Network Reliability – described HSRP’s virtual router concept for transparent failover of gateways.

- SecureW2, WPA2-Enterprise and 802.1X Simplified – affirmed that WPA2 (with 802.1X) is the gold standard for Wi-Fi security in enterprise deployments.

- Fortinet, What Is SNMP? – emphasized enabling SNMP agents on devices to integrate with monitoring systems for network management.